# Signal Processing: Filter Design Specifications

# Objective

The objective of this design is to isolate specific frequency components from a composite input signal consisting of sine waves at 100 Hz, 200 Hz, 300 Hz, and 400 Hz.

## Input Signal Composition

Signal = A₁·sin(2π·100·t) + A₂·sin(2π·200·t) + A₃·sin(2π·300·t) + A₄·sin(2π·400·t)

# Filter Design Table

| Frequency Component(s) | Filter Type | Cutoff Frequency/Frequencies |
| --- | --- | --- |
| 100 Hz | Low Pass Filter (LPF) | 150 Hz |
| 400 Hz | High Pass Filter (HPF) | 350 Hz |
| 100 Hz and 200 Hz | Low Pass Filter (LPF) | 250 Hz |
| 200 Hz | Band Pass Filter (BPF) | 150 Hz to 250 Hz |
| 300 Hz | Band Pass Filter (BPF) | 250 Hz to 350 Hz |
| 300 Hz and 400 Hz | High Pass Filter (HPF) | 250 Hz |
| 200 Hz and 300 Hz | Band Pass Filter (BPF) | 150 Hz to 350 Hz |
| 200 Hz, 300 Hz, and 400 Hz | High Pass Filter (HPF) | 150 Hz |
| 100 Hz and 400 Hz | Band Stop Filter (BSF) | 150 Hz to 350 Hz |

# Engineering Design Logic

Low Pass Filter (LPF): Used when the desired signals are at the lower end of the spectrum. The cutoff is set above the highest desired frequency but below the next unwanted frequency.

High Pass Filter (HPF): Used when the desired signals are at the upper end of the spectrum. The cutoff is set below the lowest desired frequency but above the next unwanted lower frequency.

Band Pass Filter (BPF): Used to isolate a 'middle' frequency or range. It allows a specific band to pass while blocking everything above and below that window.

Band Stop Filter (BSF): Also known as a notch filter, this is used to remove middle components. In the case of 100 Hz and 400 Hz, the BSF 'cuts out' the 200-300 Hz range to leave only the extremes.